# The 7 Layers of the OSI Model & Their Protocol Data Units (PDUs)

In computer networking, a Protocol Data Unit (PDU) is a specific unit of data that is processed and transferred at any given layer of the OSI model. As data travels down the sending stack, each layer wraps the received data with its own header (and sometimes a trailer) in a process called encapsulation. When the data travels up the receiving stack, this process is reversed (decapsulation), with each layer stripping away its respective header to read the PDU.

## Quick Reference Summary Table

| Layer Number | Layer Name | Protocol Data Unit (PDU) | Key Functions | Example Protocols & Hardware |
| --- | --- | --- | --- | --- |
| Layer 7 | Application | Data (or Message) | Human-computer interaction, network services | HTTP, HTTPS, SMTP, FTP, SSH |
| Layer 6 | Presentation | Data | Encryption, compression, formatting | SSL/TLS, JPEG, ASCII, GIF |
| Layer 5 | Session | Data | Session checkpointing, connection control | NetBIOS, SOCKS, RPC |
| Layer 4 | Transport | Segment (TCP) / Datagram (UDP) | End-to-end connections, flow control, error recovery, port addressing | TCP, UDP |
| Layer 3 | Network | Packet | Logical routing, path determination, IP addressing, device connectivity | IP (IPv4/IPv6), ICMP, Routers, Layer 3 Switches |
| Layer 2 | Data Link | Frame | Physical addressing, node-to-node transfer, error/access control | Ethernet, PPP, Switches, MAC Addresses |
| Layer 1 | Physical | Bits | Raw binary transmission over physical medium, bit synchronization | Fiber, Coaxial, Wi-Fi, Hubs, Cables |

## 💡 Essential Rules of Network Addressing & Boundaries

During network communication across multiple hops or routers, addresses and network boundaries behave according to strict fundamental rules.

### The Address Invariance Rule

- IP and Port Addresses Never Change:

  - Throughout the entire transit path from the original sender to the final receiver, the Source/Destination IP addresses and Source/Destination Port addresses remain constant.

  - This is because the IP determines the ultimate endpoint host, and the Port determines the ultimate target application on that host.

- MAC Addresses Change on Every Single Link (Hop-to-Hop):

  - The Source/Destination MAC addresses are rewritten at every router hop along the path.

  - As a packet is forwarded from one router interface to the next, the old Layer 2 Frame is stripped off, and a new Frame is created with the MAC address of the current sending interface and the MAC address of the next physical node/hop.

### The 3 Golden Rules of Network Boundaries

To determine where one network ends and another begins, apply these three core architectural rules:

- Rule 1: All interfaces connected to a router belong to DIFFERENT networks.

  - Routers are Layer 3 boundary devices. Each physical port (interface) on a router represents a completely unique subnet/logical network.

- Rule 2: All interfaces connected to a switch (or bridge) belong to the SAME network.

  - Switches and bridges are Layer 2 devices. They do not partition networks; they allow multiple end devices to share a single network domain.

- Rule 3: A single physical line/wire belongs to the SAME network.

  - Any direct cable linking two interfaces (such as host-to-switch, switch-to-switch, or router-to-router) is a single physical medium and resides within the same network space.

### LAN vs. WAN Boundaries

- LAN (Local Area Network): * This is communication inside a single local network.

  - LANs are constructed using connecting devices like switches, bridges, and hubs to link end devices (computers, printers, etc.) together.

- WAN (Wide Area Network):

  - This is communication across separate, geographically distant networks.

  - A classic WAN link is a direct point-to-point connection between two routers (often referred to as a point-to-point network).

## Detailed Breakdown of Layers and PDUs

### 7. Application Layer

- PDU: Data (sometimes called Message or Application PDU)

- Description: This layer directly interacts with software applications (like web browsers or email clients). The PDU here represents the raw user-level information before any network headers are added.

- Analogy: Writing a letter and placing it on your desk.

### 6. Presentation Layer

- PDU: Data

- Description: This layer formats, encrypts, and compresses the data to ensure it can be read by the receiving system. The PDU remains represented as "Data," but its internal syntax or encoding (e.g., translating to ASCII, encrypting with SSL) is modified here.

- Analogy: Translating the letter into a common language (e.g., English) and writing it in invisible ink for security.

### 5. Session Layer

- PDU: Data

- Description: Manages the active session between the two communicating systems (opening, maintaining, and closing connections). Like Layers 6 and 7, the PDU is still categorized broadly as "Data."

- Analogy: Making a phone call, checking if the other person is still on the line, and hanging up when finished.

### 4. Transport Layer

- PDU: Segment (for connection-oriented TCP) or Datagram (for connectionless UDP)

- Description: This is where the actual "networking" encapsulation begins. The Transport layer takes the large stream of Session Data and breaks it down into manageable chunks. It is responsible for end-to-end service delivery and application-to-application communication.

#### Key Concepts & Port Addressing

- Port Addressing: To ensure data reaches the correct application on a host, the Transport layer adds Source Port and Destination Port addresses to its header.

- Port Size & Range: A port address is a -bit binary number, which allows for a range of:

- Representation: Port addresses are represented as single decimal numbers. Examples include:

  - Port 80: Web traffic (HTTP)

  - Port 443: Secure Web traffic (HTTPS)

  - Port 23: Telnet (Remote terminal access over TCP)

  - Port 25: Simple Mail Transfer Protocol (SMTP)

#### Core Functions of TCP (Transmission Control Protocol)

- Connection Control: TCP is a connection-oriented protocol. It establishes a reliable logical connection before transmitting data using a three-way handshake (SYN  SYN-ACK  ACK) and terminates it gracefully when finished (FIN  ACK  FIN  ACK).

- Flow Control: Prevents a fast sender from overwhelming a slow receiver. TCP uses a dynamic sliding window mechanism where the receiver continuously tells the sender how much data it is currently prepared to buffer and accept.

- Error Control: Ensures 100% reliable data delivery. It numbers each byte sent using Sequence Numbers and requires the receiver to send back Acknowledgments (ACKs). If a segment is lost, damaged, or received out-of-order, TCP detects it and triggers a retransmission.

- Multiplexing & Demultiplexing: Allows multiple applications on a single device to share a single physical network connection simultaneously.

  - Multiplexing (Sender side): Gathering data segments from different application sockets, wrapping them with port headers, and passing them to the Network layer.

  - Demultiplexing (Receiver side): Examining the destination ports of incoming segments to direct them to the appropriate application processes.

- Analogy: Cutting a very long letter into numbered pages so they can be reassembled in the correct order. The port numbers are like adding the names of the specific family members to each page of the letter, so the receiving house knows exactly who gets which part of the message.

### 3. Network Layer

- PDU: Packet

- Description: The Network layer manages host-to-host connectivity across different, separate networks (inter-network communication). It takes the Segment or Datagram from Layer 4 and encapsulates it with a Network header containing logical source and destination addresses.

#### Key Concepts & Logical Addressing

- Logical Addressing (IP Address): Unlike physical MAC addresses (which are burned into hardware and cannot change), IP addresses are logical, hierarchical, and assigned dynamically or statically based on the host's location in the network.

- IP Address Size & Structure (IPv4): An IPv4 address is a -bit binary number.

  - This size allows for a maximum of:

  - Representation: It is written as a single dotted-decimal number consisting of four octets (representing  bits each) separated by periods (e.g., 192.168.1.1 or 8.8.8.8).

  - (Note: IPv6 expands this addressing space to a -bit hexadecimal format to accommodate billions of more devices).

- Routing (Communication ACROSS Networks): The process of determining the optimal path for a packet to travel from its source network to its destination network. Intermediate network devices use routing protocols and routing tables to make decisions on where to forward incoming packets next.

- Point-to-Point (WAN) Connections: While local networks (LANs) typically operate on shared, multi-access physical media, WAN connections spanning large distances often rely on dedicated Point-to-Point links. At the Network layer, packets are routed across these wide-area links (such as leased lines or point-to-point tunnels) directly between edge routers to connect geographically separate networks. The direct physical line connecting two routers forms a Point-to-Point WAN.

#### Network Entities: End Devices vs. Connecting Devices

The Network layer establishes a bridge between user hardware and the physical transport infrastructure:

- End Devices (Hosts): These are the physical systems where data originates or terminates. Examples include:

  - Computers, smartphones, and tablets.

  - Servers and network printers.

  - Smart IoT appliances.

  - Role: Every active end device must be configured with a unique logical IP address to send or receive packets over an IP network.

- Connecting Devices (Intermediary Hardware): These are network components that link distinct local area networks (LANs) together and guide traffic between them. Examples include:

  - Routers: The quintessential Layer 3 device. Routers examine the destination IP of an incoming Packet, check their routing tables, and forward it toward its destination network.

  - Layer 3 Switches: High-performance switches that combine the fast hardware-switching of Layer 2 with the logical routing capabilities of Layer 3.

- Analogy: Placing each numbered page of the letter into its own envelope and writing the sender's and receiver's mailing addresses on them. The postal trucks and sorting facilities act as the connecting devices (routers) that inspect the zip codes to determine the quickest path to delivery.

### 2. Data Link Layer

- PDU: Frame

- Description: The Data Link layer is responsible for Hop-to-Hop (Node-to-Node) delivery—handling communication inside a single network domain. Unlike Layer 3, which handles the macro routing across different networks, Layer 2 focuses purely on transmitting data reliably across a single physical link connecting two adjacent nodes.

#### The Role of the Trailer

The Data Link Layer is uniquely distinguished as the only layer in the OSI model that appends both a header and a trailer to its PDU during encapsulation.

- Why it adds a Trailer: Since the Physical layer (Layer 1) below is highly prone to noise, attenuation, and environmental interference, bits can easily get flipped or corrupted during transmission. The Data Link Layer adds a trailer containing a Frame Check Sequence (FCS)—usually a Cyclic Redundancy Check (CRC) value.

- The Verification Mechanism: The sender calculates a mathematical code based on the frame's contents and places it in the trailer. The receiver recalculates this code upon receiving the frame. If the codes match, the frame is processed; if they do not match, the receiver knows the data was corrupted and discards it immediately, avoiding the waste of higher-layer CPU resources on broken packets.

#### The 5 Core Functions of the Data Link Layer

- Framing: It takes packets from the Network Layer and wraps them into discrete "frames" by adding starting and ending bit patterns (flags) so the receiver can detect where a frame begins and ends.

- Physical Addressing: It adds physical MAC addresses to the frame header to identify the immediate sender and the immediate receiver on the local link.

- Flow Control: If the rate at which the receiver can process data is slower than the rate at which the sender is transmitting, the Data Link layer employs local flow control mechanisms to prevent buffer overflow at the adjacent node.

- Error Control: It detects and handles damaged, lost, or duplicated frames. This is primarily done using the error-detecting trailer (FCS) and acknowledgment/retransmission mechanisms on reliable links.

- Access Control (Medium Access Control): When multiple devices share a single physical communication medium (such as multiple devices on a Wi-Fi channel or a shared Ethernet hub), the Data Link Layer determines which device has control over the link and gets to transmit at any given instant (e.g., using protocols like CSMA/CD or CSMA/CA).

#### Physical MAC Addressing

- Physical MAC Address Structure: Every network interface card (NIC) has a globally unique, physical MAC (Media Access Control) address burned into its hardware.

- Size: A MAC address is  bits long.

- Representation: It is represented by  hexadecimal digits grouped in pairs ( octets) and separated by hyphens (-) or colons (:).

  - Example with hyphens: 00-1A-2B-3C-4D-5E

  - Example with colons: 00:1A:2B:3C:4D:5E

  - Organization: The first  bits ( octets) represent the OUI (Organizationally Unique Identifier, identifying the manufacturer), and the last  bits represent the unique serial number assigned by that manufacturer.

#### Key Concept: The Role of Switches and Bridges

- Single Network Domain: All interfaces of a switch or a bridge belong to the same local network (or broadcast domain).

- Switches Do Not Create Networks: A switch does not create, partition, or define separate logical networks. Instead, its sole job is to facilitate communication within a network, allowing multiple end devices to physically connect, discover one another, and exchange Layer 2 Frames. To connect or create distinct networks, a Layer 3 device (like a router) is required.

- Analogy: Giving the envelope to a local courier who adds a local routing slip containing the specific physical delivery instructions for the next stop.

### 1. Physical Layer

- PDU: Bits

- Description: The Physical layer is the bottom-most layer. It does not deal with headers, trailers, or logical segments; instead, it is responsible for the actual physical transmission of raw, unstructured Bits (s and s) over a physical medium.

#### Core Functions of the Physical Layer

- Representation of Bits: It defines how s and s are translated into physical signals (e.g., voltage levels for copper cables, light pulses for fiber optic cables, or radio wave frequencies for wireless media).

- Data Rate (Transmission Rate): Defines the number of bits sent per second (bandwidth/throughput).

- Bit Synchronization: The sender and receiver must use synchronized clocks. The Physical Layer ensures that both devices agree on the exact timing of when a bit begins and ends, preventing data misalignment.

- Interface Characteristics & Physical Topologies: * It specifies the mechanical and electrical design of physical interfaces, connectors, and cables (such as RJ-45 jacks, coaxial pins, etc.).

  - It defines how devices are arranged physically in a network (e.g., mesh, star, bus, or ring topologies).

- Transmission Mode: Defines the direction of signal flow between two devices:

  - Simplex: One-way communication only (e.g., keyboard to computer).

  - Half-Duplex: Two-way communication, but only one device can transmit at a time (e.g., walkie-talkies).

  - Full-Duplex: Simultaneous two-way communication (e.g., telephone calls).

#### Physical Layer Devices

- Hubs and Repeaters: These hardware components belong to Layer 1 because they do not read, process, or inspect any headers, MAC addresses, or IP addresses. They simply receive incoming physical electrical/optical signals (bits) on one port, amplify them, and regenerate (repeat) them out to all other ports.

- Analogy: Sending the letter's contents via Morse code flashes or telegraph pulses.